Week 7 Task 4 — Commands & Run Guide

LX2160A + LA1224 | LSDK 2108 | DPDK 22.11 | 12 May 2026 | Tessolve Semiconductors

1.  Directory Setup (Board)

Create the Week 7 workspace and copy required files from Week 6:

# Create Week 7 directory structure

mkdir -p /root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/src

mkdir -p /root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/scripts

mkdir -p /root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/logs

# Copy Makefile from Week 6

cp /root/dpdk_workspace/week6/Task_3_bbdev_Enqueue/src/Makefile \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/src/

# Copy build and run scripts

cp /root/dpdk_workspace/week6/Task_3_bbdev_Enqueue/scripts/build.sh \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/scripts/

cp /root/dpdk_workspace/week6/Task_3_bbdev_Enqueue/scripts/run.sh \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/scripts/

# Copy setup script

cp /root/dpdk_workspace/week6/Task_3_bbdev_Enqueue/setup_state.sh \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/

# Rename binary in Makefile from kyocera_pmd_init to kyocera_pmd_dq

sed -i 's/kyocera_pmd_init/kyocera_pmd_dq/g' \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/src/Makefile

sed -i 's/kyocera_pmd_init/kyocera_pmd_dq/g' \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/scripts/build.sh

2.  SCP main.c to Board (Mac)

Transfer the Task 4 main.c from Mac to board:

scp main.c user@192.168.10.11:~/main.c

# On board: copy to src directory

cp /home/user/main.c \

/root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath/src/main.c

3.  Build (Board)

cd /root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath

./setup_state.sh

export DPRC=dprc.2

./scripts/build.sh 2>&1 | tail -5

Expected output:

Built: .../src/kyocera_pmd_dq

NOTE: If EXIT_FAILURE undeclared error: add #include <stdlib.h> to main.c and rebuild.

4.  Run Sequence

Three windows required: Board, LA1224 UART, Mac.

Window 2 — LA1224 UART (arm modem first)

test bbdev_ipc1 0x3f 1

Window 1 — Board (start app immediately after)

cd /root/dpdk_workspace/week7/Task_4_Dequeue_ReturnPath

rm -f /mnt/huge/* && rm -rf /var/run/dpdk/kpmd/ 2>/dev/null

./src/kyocera_pmd_dq -l 0-3 -n 1 --file-prefix=kpmd \

--vdev=baseband_la12xx \

2>&1 | tee logs/week7_task4_$(date +%Y%m%d_%H%M%S).log

Wait for this line before sending packets:

KPMD: APP: waiting on DPMAC.1 RX...

Window 3 — Mac (send packets after app is ready)

sudo python3 /tmp/send_udp.py

Window 4 — Mac (capture return traffic)

sudo tcpdump -i en0 udp port 4001 -nn -c 60

5.  Scapy Scripts

5.1  Smoke Test — 60 Packets

cat > /tmp/send_udp.py << 'EOF'

from scapy.all import *

BOARD_MAC = "00:04:9f:08:02:78"

MAC_IP    = "192.168.10.20"

BOARD_IP  = "192.168.10.11"

IFACE     = "en0"

COUNT     = 60

print(f"Sending {COUNT} UDP packets...")

pkt = Ether(dst=BOARD_MAC) / IP(src=MAC_IP, dst=BOARD_IP) \

/ UDP(sport=1234, dport=5678) \

/ Raw(b"KyoceraUDP" + b"X" * 166)

sendp(pkt, iface=IFACE, count=COUNT, inter=0.05, verbose=False)

print(f"Done! Sent {COUNT} packets")

EOF

5.2  Full Run — 5000 Packets (controlled pacing)

cat > /tmp/send_udp.py << 'EOF'

from scapy.all import *

import time

BOARD_MAC = "00:04:9f:08:02:78"

MAC_IP    = "192.168.10.20"

BOARD_IP  = "192.168.10.11"

IFACE     = "en0"

COUNT     = 5000

print("Waiting 20s for app to be ready...")

time.sleep(20)

print(f"Sending {COUNT} UDP packets...")

pkt = Ether(dst=BOARD_MAC) / IP(src=MAC_IP, dst=BOARD_IP) \

/ UDP(sport=1234, dport=5678) \

/ Raw(b"KyoceraUDP" + b"X" * 166)

sendp(pkt, iface=IFACE, count=COUNT, inter=0.06, verbose=False)

print(f"Done! Sent {COUNT} packets")

EOF

6.  Why We Slow Down the Packet Flow

There are two reasons we use inter=0.06s (one packet every 60ms) instead of full burst speed:

Reason 1 — The 60-op Batch Ceiling

The LA1224 FECA hardware has a known bug inside fill_feca_desc_enc() in the NXP la12xx PMD. After approximately 60-70 LDPC encode operations the modem raises a QDMA Descriptor Format Error and stops processing. This is an open NXP escalation.

Our workaround is a strict drain-then-submit loop with a 60-op batch ceiling:

Submit batch of 60 ops

--> Wait for all 60 to complete (drain)

--> Then submit next batch of 60

The drain takes approximately 3 seconds per batch. If scapy sends packets faster than the drain completes, new packets pile up in the RX queue. When the app collects them it can still only submit 60 at a time and the rest become ops_enq_fail.

| Send rate | Drain time | Result |
| --- | --- | --- |
| inter=0.0002s (full burst) | 3s per batch | Overflow — ops_enq_fail=2000+ |
| inter=0.06s (controlled) | 3s per batch | Balanced — ops_enq_fail=0 |

Reason 2 — App Startup Timing

The app starts its poll timer from boot, not from when the first packet arrives. Initialisation, IPC handshake, and 10G link-up take approximately 15-20 seconds. If scapy starts immediately, some poll window is consumed before packets arrive.

The 20-second sleep at the start of the 5000-packet script ensures the app is fully ready and polling before any packets are sent.

What changes after NXP fix

Once NXP delivers a PMD patch fixing the ring wrap-around bug, the batch ceiling is removed. Full burst speed becomes possible:

# After NXP fix -- target behaviour

inter=0.0002s, COUNT=5000

--> ops_enq=5000  ops_enq_fail=0  tx_sent=5000

7.  Expected Final Output

Host (Board)

KPMD: DONE: rx_total=7512 rx_test=5000

ops_enq=5000 ops_enq_fail=0

ops_deq=5000 ops_failed=0

tx_sent=5000 tx_drop=0

Mac tcpdump

192.168.10.11.5001 > 192.168.10.20.4001: UDP, length 355

60 packets captured | 0 packets dropped by kernel

Platform: NXP LA1224-RDB RevC | LSDK 2108 | DPDK 22.11 | May 2026