Week 7 Task 4 — LDPC Dequeue & Return Path: Test Report

LX2160A + LA1224 | LSDK 2108 | DPDK 22.11 | 12 May 2026 | Tessolve Semiconductors

1.  Overview

| Field | Details |
| --- | --- |
| Customer | Kyocera SLD Laser, Inc. |
| Platform | NXP LA1224-RDB RevC (LX2160A host + LA1224 modem) |
| Software | LSDK 2108 / DPDK 22.11 / librte_baseband_la12xx |
| Binary | kyocera_pmd_dq |
| Test date | 12 May 2026 |
| Task | Week 7 / Task 4 — Dequeue and Return-Path Bring-up |
| Result | PASS — 5000/5000 end-to-end |

2.  Pipeline (Task 4 Full Flow)

Mac (scapy)

--> UDP packets --> Board RX (DPMAC.1)

--> Kyocera signature filter (dport=5678)

--> LDPC encode via LA1224 FECA      [Task 3]

--> Dequeue completed encode ops      [NEW Task 4]

--> Re-encap: Eth/IPv4/UDP headers   [NEW Task 4]

--> TX return path (DPMAC.1)         [NEW Task 4]

--> Mac receives encoded frames on port 4001

3.  LDPC Parameters (Kyocera Locked 05-May-2026)

| Parameter | Value | Notes |
| --- | --- | --- |
| Base Graph | BG1 | Kyocera confirmed |
| Lifting size z_c | 64 | Constant across bandwidth |
| Modulation q_m | 2 (QPSK) | MCS-1 starting point |
| Code rate | 1/2 | E = 2 x K = 2816 bits |
| RV | 0 | ARQ handled by Ethernet/IP stack |
| Mode | Code Block | CB mode confirmed |
| CRC | 24-bit CB-CRC | Appended by LA1224 firmware |
| Output size | 355 bytes | (E+7)/8 + 3 bytes CB-CRC |

4.  Test Results

4.1  Smoke Test — Single 60-op Batch

| Counter | Result | Status |
| --- | --- | --- |
| rx_test | 60 | PASS |
| ops_enq | 60 | PASS |
| ops_enq_fail | 0 | PASS |
| ops_deq | 60 | PASS |
| ops_failed | 0 | PASS |
| tx_sent | 60 | PASS |
| tx_drop | 0 | PASS |

4.2  Return Traffic Verification (tcpdump on Mac)

60 encoded UDP frames received on Mac Studio en0, port 4001:

192.168.10.11.5001 > 192.168.10.20.4001: UDP, length 355

60 packets captured | 0 packets dropped by kernel

Length 355 bytes = 352 bytes encoded payload + 3 bytes CB-CRC. Correct.

4.3  Final Run — 5000 Packets (PASS)

5000 UDP test packets sent from Mac at inter=0.06s (one packet every 60ms to respect the 60-op batch ceiling). App polled for up to 600 seconds.

| Counter | Result | Status |
| --- | --- | --- |
| rx_total | 7512 | PASS |
| rx_test | 5000 | PASS |
| ops_enq | 5000 | PASS |
| ops_enq_fail | 0 | PASS |
| ops_deq | 5000 | PASS |
| ops_failed | 0 | PASS |
| tx_sent | 5000 | PASS |
| tx_drop | 0 | PASS |
| op_alloc_drops | 0 | PASS |
| out_alloc_drops | 0 | PASS |

5000/5000 packets encoded and returned to Mac. Zero failures. Zero drops. Full pipeline proven end-to-end.

5.  Pass / Fail Summary

| Test Item | Result |
| --- | --- |
| IPC handshake + FECA ready | PASS |
| 10G link UP, RX path working | PASS |
| LDPC encode ops complete (status==0) | PASS |
| Dequeue drain: deq == enq every batch | PASS |
| Re-encapsulation: Eth/IPv4/UDP headers correct | PASS |
| TX return path: frames transmitted | PASS |
| Mac receives return frames on port 4001 | PASS |
| Return frame length: 355 bytes (352 + 3 CRC) | PASS |
| ops_failed = 0 across all 5000 ops | PASS |
| tx_drop = 0 across all 5000 ops | PASS |
| 5000/5000 ops at controlled pacing | PASS |
| 5000/5000 ops at full burst rate | PARTIAL — blocked on NXP PMD fix |

6.  Known Limitation

The 60-op batch ceiling is a workaround for the Task 3 QDMA descriptor format error that triggers at op ~60-87. This is a confirmed bug inside fill_feca_desc_enc() in the la12xx PMD. An NXP escalation is open.

At full burst rate (inter=0.0002s) the modem ring overflows and ops_enq_fail rises. At controlled pacing (inter=0.06s) the ring drains between batches giving 5000/5000 with zero failures.

| Scenario | ops_enq_fail | tx_sent |
| --- | --- | --- |
| Full burst (inter=0.0002s) | ~2000+ | ~2300 |
| Controlled pacing (inter=0.06s) | 0 | 5000 |
| After NXP PMD fix (target) | 0 | 5000 |

7.  Next Steps

•  Await NXP PMD patch (fill_feca_desc_enc() ring wrap-around fix) to enable full burst operation.

•  Byte-correctness check: capture pcap on Mac, extract first 3 encoded payloads (352 bytes), compare against reference LDPC encoder with locked parameters.

•  Implement adaptive MCS (Tier 1): CLI flag --mcs=N with lookup table for q_m and E per 802.11 MCS table.

•  Once NXP fix received: remove NB_BATCH_CAP ceiling, retest at 5000 ops full burst, target enq_fail=0 at inter=0.0002s.

Platform: NXP LA1224-RDB RevC | LSDK 2108 | DPDK 22.11 | May 2026