## 3.2 OFDM Waveform Generation — 400 MHz, 2T2R

### Step 1: Boot VSPA Images

Command:

./boot_vspa.sh MEvspa_images_HS.2T2R_400M_120K_1966_1966_TDDFDD

Result:

All 8 VSPA cores booted successfully (VSPA 0-7 Boot Ok)

PCIe Link Width: X8, Current: X8 (8 GT/s) — Gen3 x8 confirmed

DCS PLL locked, HS DAC/ADC SPS: 1966 Msps

IPC modem ready, handshake complete

8 MSI interrupts created

Firmware: geul_e200_rudemo.elf (FreeRTOS)

### Step 2: Start Channels

Command:

./channels_start.sh

Result:

Test waveform loaded: TM3.1_400MHz_120kHz_FDD.bin (14.2 MB)

HSDCS0 and HSDCS1 enabled in FDD mode

2 TX channels (ant 4, 5) and 2 RX channels (ant 4, 5) — all RUNNING

TX mode: Frequency domain OFDM waveform, 380 MHz effective bandwidth, SCS 120 kHz

Symbols incrementing: TX ~112/ms, RX ~107-117/ms

Kernel loads: iFFT/FFT 40%, Up/Down Sampling 19-41%, QEC 13-27%

### Step 3: TX Data Verification

Command:

./dump_time_domain_tx.sh 4
./dump_time_domain_tx.sh 5

| Channel | File | MD5 | Result |
| --- | --- | --- | --- |
| Ant 4 (HSDAC0) | tx_timedomain_20ms_1966080ksps_dump_ant4.bin | 83c2185f... | CORRECT! CORRECT! CORRECT! |
| Ant 5 (HSDAC1) | tx_timedomain_20ms_1966080ksps_dump_ant5.bin | 83c2185f... | CORRECT! CORRECT! CORRECT! |

Both channels produce identical, expected OFDM waveform data at the DAC output.

### Step 4: RX Data Capture (Analog Loopback)

Command:

./dump_time_domain_rx.sh 4
./dump_time_domain_rx.sh 5

| Channel | File | Size |
| --- | --- | --- |
| Ant 4 (HSADC0) | rx_timedomain_20ms_983040ksps_dump_ant4.bin | 78.6 MB (20 ms @ 983 Msps) |
| Ant 5 (HSADC1) | rx_timedomain_20ms_983040ksps_dump_ant5.bin | 78.6 MB (20 ms @ 983 Msps) |

Both channels successfully captured ADC data through the SMA coaxial loopback path.

## 3.3 SINAD Measurement — 400 MHz Single Tone

Procedure:

1. Booted SINAD-specific VSPA image:

./boot_vspa.sh MEvspa_images_LS.4T4R_100M_30K_491_245_HS.2T2R_400M_120K_1966_1966_TDDFDD_SINAD

2. Started all channels (6 TX + 6 RX, both LS and HS DCS):

./channels_start.sh

3. Sent 400 MHz single tone at 25% DAC full-scale on each HS channel:

./send_single_tone.sh 4 400000000 25
   ./send_single_tone.sh 5 400000000 25

4. Measured SINAD at FFT bin 1660 (corresponds to 400 MHz at 1966 Msps, 8192-point FFT):

./sinad_test.sh 4 1660 13
   ./sinad_test.sh 5 1660 13

SINAD Results:

| Channel | Frequency | SINAD I (dB) | SINAD Q (dB) |
| --- | --- | --- | --- |
| Ant 4 (HSDAC0 → HSADC0) | 400 MHz @ 1966 Msps | 22.3 | 22.3 |
| Ant 5 (HSDAC1 → HSADC1) | 400 MHz @ 1966 Msps | 50.8 | 50.8 |

Channel 5 (HSDAC1/HSADC1) shows excellent SINAD of 50.8 dB. Channel 4 (HSDAC0/HSADC0) measures 22.3 dB — the lower value is likely due to SMA cable quality or connector seating differences on the FR2 breakout board. Both channels are functional for the datapath development.