DATAPATH DRIVER DEVELOPMENT

| Project | Datapath Driver Development |
| --- | --- |
| Customer | Kyocera SLD Laser, Inc. |
| Prepared by | Tessolve Semiconductors |
| Date | 15th April 2026 |
| Version | V 0.1 |
| Classification | CONFIDENTIAL AND PROPRIETARY |

# Content:

1. Overview	3

2. Test Environment	3

Console Setup	3

Physical setup	3

Serial Console Setup	3

3. Test 1: 10G Ethernet Throughput (iperf)	5

Objective	5

Procedure	5

Step 1: Enable Jumbo Frames (MTU 9000)	6

Step 2: TCP Buffer Tuning	6

Step 3: Increase TX Queue Length	6

Step 4: CPU Performance Mode	7

Step 5: iPerf Testing	7

Result	7

4. Test 2: LDPC Processing via bbdev	8

Objective	8

Procedure	8

Result	9

5. Test 3: PCIe IPC Latency Benchmark	10

Objective	10

Procedure	10

Result	10

6. Overall Results Summary	11

# 1. Overview

This document records the execution and results of three hardware validation tests performed on the NXP LA1224-RDB platform during Week 03. These tests validate the three building blocks of the datapath: Ethernet connectivity, LDPC hardware acceleration, and PCIe communication latency.

| Test | Validates | Interface |
| --- | --- | --- |
| Test 1 | 10G Ethernet throughput | Mac Studio to LX2160A |
| Test 2 | LDPC encode/decode (FECA) | LX2160A to LA1224 (PCIe) |
| Test 3 | Raw PCIe IPC latency | LX2160A to LA1224 (PCIe) |

# 2. Test Environment

| Component | Detail |
| --- | --- |
| Development Board | NXP LA1224-RDB Rev-C (LA1235 B0, HSDCS_FULL) |
| Host Processor | LX2160A, 16-core ARMv8-A |
| Kernel | Linux 5.15.71-rt51 (PREEMPT_RT) |
| SDK | NXP LA12xx-SDK v3.2 / LSDK 2108 |
| LA1224 Firmware | FreeRTOS V10.6.0, geul_e200_rudemo.elf |
| PCIe Link | Gen3 x8 (8 GT/s) |
| Test Host | Mac Studio (10G Ethernet + 2x FTDI USB Serial) |

## Console Setup

Three concurrent sessions were used throughout testing:

Window 1: LX2160A UART (serial, 115200 baud) for boot visibility and recovery

Window 2: LA1224 UART (serial, 115200 baud) for FreeRTOS console, arming modem side tests

Window 3: SSH to LX2160A for primary command execution

## Physical setup

Two USB-serial (FTDI) cables connecting Mac Studio to the two UART consoles: one for LX2160A, one for LA1224.

10G Ethernet cable from Mac Studio 10G port to LA1224-RDB J4/J5.

SSH session to LX2160A over the 10G link for running host commands (bbdev setup, testbbdev, bbdev_raw_app).

## Serial Console Setup

Step 1 - Enumerate serial devices

Prompt:
ls /dev/cu.usb* /dev/tty.usb*

Output:

/dev/cu.usbserial-FTELCW6K   /dev/cu.usbserial-FTERJ2D5
/dev/tty.usbserial-FTELCW6K  /dev/tty.usbserial-FTERJ2D5

Step 2 - Identify the LX2160A UART

Prompt:

screen /dev/cu.usbserial-FTELCW6K 115200

Output:

A Linux root shell prompt appeared after pressing Enter — this confirms the cable is connected to the LX2160A UART.

Step 3 - Identify the LA1224 UART

Prompt:

screen /dev/cu.usbserial-FTERJ2D5 115200

Output:

Blank screen, no prompt, no boot log. This is the LA1224 UART; it stays silent until the yami driver is loaded on the LX2160A side and brings up the LA1224 firmware. Expected behavior, not a failure.

# 3. Test 1: 10G Ethernet Throughput (iperf)

## Objective

Validate the 10G Ethernet link between the host system (MAC Studio) and LX2160A using iperf3.

## Procedure

Serial access to LX2160A:

Minicom –D /dev/cu.usbserial-FTERJ2D5 –b 115200

Logged	in as root:

sudo su -

Iperf3 installed on LX2160A:

Temporarily plugged Ethernet into office network for DHCP/internet:

Dhclient –v eth1

apt update && apt install –y iperf3

Confirmed: iperf 3.7 installed.

Iperf3 installed on Mac:

Brew install iperf3

Confirmed: iperf 3.21 installed

On LX2160A (Window 3):

source /usr/local/dpdk/dpaa2/dynamic_dpl.sh dpmac.3

ls-addni dpmac.4
ifconfig eth2 192.168.10.11 netmask 255.255.255.0 up

ethtool	eth2 | grep –E "Speed|Link"

#Output:

Speed 10000Mb/s Link

detected: yes

Run :  iperf3 –s

On Host system (MAC Studio):

sudo ifconfig en0 192.168.10.2 netmask 255.255.255.0 up

Ping -c 4 192.168.10.11

#Output:

4 packets transmitted, 4 packets received, 0.0% packet loss round-trip min/avg/max/stddev=0.531/0.770/1.141/0.229 ms

Run: iperf3 -c 192.168.10.11 -t 10

Initial throughput: ~6.4 Gbps

# Step 1: Enable Jumbo Frames (MTU 9000)

Increased MTU from 1500 to 9000 to reduce packet overhead and improve throughput.
LX2160:

ip link set eth2 mtu 9000

Mac:

sudo ifconfig en0 mtu 9000

# Step 2: TCP Buffer Tuning

Increased RX/TX buffers to handle high speed traffic.
LX2160:

sysctl -w net.core.rmem_max=134217728
sysctl -w net.core.wmem_max=134217728
sysctl -w net.ipv4.tcp_rmem='4096 87380 134217728'
sysctl -w net.ipv4.tcp_wmem='4096 65536 134217728'
sysctl -w net.core.netdev_max_backlog=250000

Mac:
sudo sysctl -w net.inet.tcp.recvspace=4194304
sudo sysctl -w net.inet.tcp.sendspace=4194304

# Step 3: Increase TX Queue Length

Increased transmit queue to handle burst traffic:

LX2160:

ip link set eth2 txqueuelen 10000

# Step 4: CPU Performance Mode

Set the CPU governor to perform maximum processing:
LX2160:

for i in /sys/devices/system/cpu/cpu*/cpufreq/scaling_governor; do echo performance > $i; done

# Step 5: iPerf Testing

Validated throughput after optimization.

LX2160:

Run: iperf3 –s

Mac:

iperf3 -c 192.168.10.11 -P 4 -t 30
iperf3 -c 192.168.10.11 -P 8 -t 30

# Result

| Metric | Value |
| --- | --- |
| Single Stream | ~8.6 Gbps |
| Multi Stream | ~8.7 Gbps |
| Packet Loss | Zero |

Verdict: PASSED

# 4. Test 2: LDPC Processing via bbdev

## Objective

Validate LDPC encode/decode on the LA1224 FECA engine via the DPDK bbdev framework over PCIe.

Loading the yami PCIe Driver (Critical for Tests 2 & 3)

The yami kernel module is the LX2160A-side PCIe driver for the LA1224. Loading it triggers firmware download, VSPA core boots, and the FreeRTOS image coming alive on the LA1224. Without yami, the LA1224 UART stays silent, and no bbdev or IPC tests can proceed.

## Procedure

Step 1: Clean reset (Window 3)

rmmod yami

rm -f /dev/hugepages/*

cat /proc/meminfo | grep -i huge           # expect HugePages_Free: 2

echo 1 > /sys/bus/pci/rescan

sleep 2

insmod $(find /lib/modules/ - name yami.ko) scratch_buf_size=0x20000000 scratch_buf_phys_addr=0x2360000000

hsdcs_enable=1

sleep 12

lsmod | grep yami                           # should show: yami  188416  0

dmesg | tail -20                            # confirm 'Modem Id 0: Probed OK'

Step 2: Arm LA1224 (Window 2)

test bbdev_ipc1 0x3f 1

Output:

[Test Framework] Iteration => 1 Test Name => bbdev_ipc1

feca_dev_open: DEV already open

bbdev_ipc_test: ==> Processing queues

Wait for "Processing queues" message before proceeding.

Step 3: Prepare test vectors (Window 3)

cd /usr/local/dpdk/
ln -sf dpdk/app/test-bbdev/test_vectors/ldpc_dec_default.data test_vector_0.data
ln -sf dpdk/app/test-bbdev/test_vectors/ldpc_enc_default.data test_vector_1.data

Step 4: Run bbdev test (Window 3)

testbbdev --vdev=bbdev_la12xx -c 0x3 -- -n 4 -f 2 -b 4 -s 270000

## Result

| Metric | Value |
| --- | --- |
| Decode Throughput | 17.88 Mbps |
| Encode Throughput | 21.38 Mbps |
| Latency per Packet | 10 to 12 us |
| Test Status | ALL PASSED |

Verdict: PASSED

# 5. Test 3: PCIe IPC Latency Benchmark

## Objective

Measure raw PCIe DMA round trip latency between LX2160A and LA1224 without any LDPC processing overhead.

## Procedure

Step 1: Clean reset and mount hugepages (Window 3)

rmmod yami
echo 1 > /sys/bus/pci/devices/0002\:00\:00.0/rescan
insmod /lib/modules/$(uname -r)/extra/yami.ko \

scratch_buf_size=0x20000000 scratch_buf_phys_addr=0x2360000000

sleep 12

dmesg | tail -20                    # confirm 'Modem Id 0: Probed OK'
mkdir -p /dev/hugepages
mount -t hugetlbfs none /dev/hugepages

Step 2: Arm LA1224 for raw IPC (Window 2)

test bbdev_ipc_raw 0x3f 1

Wait for "Processing queues" message.

Step 3: Run benchmark (Window 3)

bbdev_raw_app --vdev=bbdev_la12xx

## Result

| Metric | Value |
| --- | --- |
| One Way Latency | ~2.93 us |
| Round Trip Latency | ~4.71 us |
| Iterations | 10,000 |
| Test Status | ALL PASSED |

Verdict: PASSED

Sub 5μs round trip confirms that the PCIe Gen3 x8 link is low latency and well within the 200 μs p99 target for the datapath driver.

# 6. Overall Results Summary

| Test | Validates | Key Numbers | Result |
| --- | --- | --- | --- |
| Test 1 | 10G Ethernet (Mac to LX2160A) | ~8.6 Gbps single stream, zero loss ~8.7 Gbps multi stream, zero loss | PASSED |
| Test 2 | LDPC encode/decode over bbdev | DEC 17.88 Mbps, ENC 21.38 Mbps | PASSED |
| Test 3 | Raw PCIe DMA latency | One way 2.93 us, round trip 4.71 us | PASSED |

All three hardware validation tests have been passed. The Ethernet front end, PCIe communication pipe, and LDPC hardware accelerator are confirmed operational. The platform is ready for DPDK driver development.

Document Approval

| Role | Name | Date |
| --- | --- | --- |
| Created By | Nandini & Mary | 13-04-2026 |
| Reviewed By | Aswanth | 14-04-2026 |
| Approved By | Deepak | 15-04-2026 |