LX2160 / LA1224 Baseband Platform

DPDK Build Environment & EAL Port Init

Week 04 Execution Report

| Project | Datapath Driver Development |
| --- | --- |
| Customer | Kyocera SLD Laser, Inc. |
| Prepared by | Tessolve Semiconductors |
| Date | 22nd April 2026 |
| Version | V 0.1 |
| Classification | CONFIDENTIAL AND PROPRIETARY |

Table of Contents

1. Overview	3

2. Environment	3

3. Task 2: Build Environment Setup	3

3.1 Verify 1 GB hugepages	3

3.2 Verify DPRC bound to vfio-fsl-mc	3

3.3 Run testpmd probe-only	4

3.4 Run testpmd TX sanity check	4

4. Task 3: EAL and Port Init Application	5

4.1 Application structure	5

4.2 Build the application	5

4.3 Run the application	5

5. Issues Encountered and Resolutions	7

Issue 1: pkg-config cannot find libdpdk	7

Issue 2: OpenSSL symbol errors when linking libdpdk.a	7

Issue 3: Missing dpaax_iova_table symbols	8

Issue 4: Makefile empty variable name error	8

6. Final Result	9

7. Summary	9

# 1. Overview

This document records the Week 04 execution on the NXP LA1224-RDB platform. The goal was to prepare the DPDK build environment on the LX2160A host and develop the first skeleton application that initializes DPDK EAL, configures the DPAA2 10G Ethernet port, creates a hugepage-backed mempool, sets up RX and TX queues, and runs a 30-second RX poll loop to confirm the datapath is live.

This work forms the foundation for the full DPDK driver application that will bridge the 10G Ethernet interface and the LA1224 baseband accelerator over PCIe via the bbdev framework.

# 2. Environment

| Component | Detail |
| --- | --- |
| Target | NXP LX2160A host, LA1224-RDB Rev-C board |
| OS | Linux 5.15.71 RT (NXP LSDK 2004 rootfs) |
| DPDK Version | 19.11-qoriq (NXP fork) |
| DPDK Location | /usr/local/include/dpdk, /usr/local/lib/librte_*.a |
| Workspace | /root/dpdk_workspace/Task_3_EAL_Port_Init/ |
| Peer | Mac Studio over 10G Ethernet for traffic generation |

# 3. Task 2: Build Environment Setup

## 3.1 Verify 1 GB hugepages

Command:

grep Huge /proc/meminfo

Output:

HugePages_Total:       2
HugePages_Free:        2
HugePages_Rsvd:        0
Hugepagesize:    1048576 kB
Hugetlb:         2097152 kB

Result: PASSED. 2 x 1 GB hugepages available.

## 3.2 Verify DPRC bound to vfio-fsl-mc

Command:

readlink /sys/bus/fsl-mc/devices/dprc.2/driver | xargs basename

Output:

vfio-fsl-mc

Result: PASSED. DPRC is correctly bound to the VFIO driver.

## 3.3 Run testpmd probe-only

Command:

testpmd -c 0xf -n 1 -- -i --nb-cores=3 --txd=2048

At the testpmd> prompt:

testpmd> show port info 0

Output (abbreviated):

Infos for port 0
MAC address: 00:04:9F:08:02:78
Device name: dpni.2
Driver name: net_dpaa2
Link status: up
Link speed: 10000 Mbps
Link duplex: full-duplex
Max possible RX queues: 128
Max possible TX queues: 16

Result: PASSED. DPDK successfully probed the DPAA2 10G port at full 10 Gbps link.

## 3.4 Run testpmd TX sanity check

At the testpmd> prompt:

testpmd> set fwd txonly
testpmd> start

After one minute, check stats:

testpmd> show port stats all

Output:

RX-packets: 8          RX-missed: 0          RX-bytes:  1232
RX-errors:  0
RX-nombuf:  0
TX-packets: 825,333,936  TX-errors: 0          TX-bytes: 52,821,373,376
Throughput (since last show):
Rx-pps:            0       Rx-bps:            0
Tx-pps:    1,349,622       Tx-bps:    691,006,488

Result: PASSED. 825 million packets transmitted with zero errors over 1 minute. DPDK owns the NIC and can drive it at high packet rate.

# 4. Task 3: EAL and Port Init Application

## 4.1 Application structure

Source file: src/main.c. The application performs the following steps:

Initialize EAL (rte_eal_init)

Enumerate DPAA2 Ethernet ports (rte_eth_dev_count_avail)

Create hugepage-backed mbuf mempool (rte_pktmbuf_pool_create)

Configure port (rte_eth_dev_configure, rte_eth_rx_queue_setup, rte_eth_tx_queue_setup)

Start port and enable promiscuous mode

Run 30-second RX poll loop (rte_eth_rx_burst)

Print RX statistics and clean up

## 4.2 Build the application

Command:

cd /root/dpdk_workspace/Task_3_EAL_Port_Init/
chmod +x scripts/*.sh
./scripts/build.sh

Output:

DPDK headers: /usr/local/include/dpdk/
DPDK libs:    /usr/local/lib/
rm -f kyocera_pmd_init *.o
cc -O3 -g -Wall -Wextra ... -o kyocera_pmd_init ...

Built: /root/dpdk_workspace/Task_3_EAL_Port_Init/src/kyocera_pmd_init

Result: PASSED. Binary kyocera_pmd_init produced successfully.

## 4.3 Run the application

Command:

export DPRC=dprc.2
./src/kyocera_pmd_init -l 0-3 -n 1 --file-prefix=kpmd

MAC:
 
   iperf3 -c 192.168.10.11 -u -b 1G -t 20 2>/dev/null &

Output:

EAL: Detected 16 lcore(s)

EAL: Detected 1 NUMA nodes

EAL: Multi-process socket /var/run/dpdk/kpmd/mp_socket

fslmc: Skipping invalid device (power)

EAL: Selected IOVA mode 'VA'

EAL: Probing VFIO support...

EAL: VFIO support initialized

PMD: dpni.3: netdev created, connected to dpmac.4

KPMD: Found 1 Ethernet port(s).

KPMD: Port 0: driver=net_dpaa2 max_rxq=128 max_txq=16 rx_offload_capa=0xe6a4f tf

KPMD: Port 0 MAC 00:04:9f:08:02:78

KPMD: Port 0 link DOWN speed 10000 Mbps duplex full

KPMD: Polling port=0 q=0 for 30 s on lcore 0 (Ctrl-C to exit early)...

KPMD: Done. RX mbufs=86 | ipackets delta=86 ibytes delta=36025 imissed delta=0 0

Result: PASSED. Application initialized DPDK EAL, configured the DPAA2 port, created the mbuf mempool, set up RX and TX queues, and received 86 packets (36,025 bytes) over the 30-second polling window with zero errors and zero drops.

# 5. Issues Encountered and Resolutions

## Issue 1: pkg-config cannot find libdpdk

Error:

Package libdpdk was not found in the pkg-config search path.
No package libdpdk found

Root cause: The NXP LSDK 2004 DPDK build does not install a libdpdk.pc file. The DPDK headers are at /usr/local/include/dpdk/ and libraries are at /usr/local/lib/librte_*.a.

Resolution: Updated the Makefile to use explicit include and library paths instead of pkg-config.

Fix applied in Makefile:

DPDK_INC := /usr/local/include/dpdk
DPDK_LIB := /usr/local/lib
CFLAGS += -I$(DPDK_INC)
LDFLAGS += -L$(DPDK_LIB)

## Issue 2: OpenSSL symbol errors when linking libdpdk.a

Error (abbreviated):

librte_pmd_openssl.a(rte_openssl_pmd.o): undefined reference to EVP_CIPHER_CTX_new
librte_pmd_openssl.a: undefined reference to EVP_aes_128_gcm
librte_pmd_openssl.a: undefined reference to HMAC_CTX_new
librte_pmd_openssl.a: undefined reference to RSA_new, DH_new, DSA_SIG_new
(hundreds of similar errors)
collect2: error: ld returned 1 exit status

Root cause: The initial Makefile used --whole-archive -ldpdk --no-whole-archive, which forced the linker to include every static library in the DPDK bundle. This pulled in librte_pmd_openssl.a, which requires libssl and libcrypto symbols that are not linked.

Resolution: Replaced the bulk libdpdk link with selective linking. Only the PMDs that require constructor registration (dpaa2, fslmc bus, mempool_dpaa2) are wrapped in --whole-archive. All other DPDK libraries are linked normally so the linker resolves only the symbols actually referenced by main.c.

Fix applied in Makefile:

PMD_LIBS := \
  -Wl,--whole-archive \
  -lrte_pmd_dpaa2 \
  -lrte_bus_fslmc \
  -lrte_mempool_dpaa2 \
  -Wl,--no-whole-archive

CORE_LIBS := -lrte_ethdev -lrte_mbuf -lrte_mempool -lrte_mempool_ring \
  -lrte_ring -lrte_net -lrte_pci -lrte_bus_pci -lrte_bus_vdev \
  -lrte_kvargs -lrte_cmdline -lrte_timer -lrte_hash -lrte_eal

## Issue 3: Missing dpaax_iova_table symbols

Error:

librte_bus_fslmc.a: undefined reference to dpaax_iova_table_populate
librte_bus_fslmc.a: undefined reference to dpaax_iova_table_depopulate
librte_mempool_dpaa2.a: undefined reference to dpaax_iova_table_update
collect2: error: ld returned 1 exit status

Root cause: librte_bus_fslmc and librte_mempool_dpaa2 depend on librte_common_dpaax, which was not in the link list.

Resolution: Added -lrte_common_dpaax to CORE_LIBS in the Makefile.

Fix applied:

CORE_LIBS := ... -lrte_hash -lrte_common_dpaax -lrte_eal

## Issue 4: Makefile empty variable name error

Error:

Makefile:6: *** empty variable name.  Stop.

Root cause: While editing the Makefile in nano, leading spaces were inadvertently added to variable assignment lines (for example "  SRCS := main.c" instead of "SRCS := main.c"). Make requires variable assignments to start at column 1 with no leading whitespace.

Resolution: Recreated the Makefile with proper formatting (no leading whitespace on variable lines, single TAB character on rule body lines) and copied it to the target.

# 6. Final Result

| Check | Result |
| --- | --- |
| 1 GB hugepages configured | PASSED |
| DPRC bound to vfio-fsl-mc | PASSED |
| testpmd probe (10G link UP) | PASSED |
| testpmd TX sanity (825M packets, 0 errors) | PASSED |
| kyocera_pmd_init builds cleanly | PASSED |
| kyocera_pmd_init runs and configures port | PASSED |
| RX poll loop captures 86 packets, 0 errors, 0 drops | PASSED |

Week 04 objectives are fully met. The LX2160A build environment is ready for DPDK application development, and the first skeleton application is in place and validated. The foundation is complete for the Week 05 bbdev integration work.

# 7. Summary

Commands that were used successfully:

# Verify hugepages
grep Huge /proc/meminfo

# Verify DPRC binding
readlink /sys/bus/fsl-mc/devices/dprc.2/driver | xargs basename

# Run testpmd
testpmd -c 0xf -n 1 -- -i --nb-cores=3 --txd=2048
# at prompt:
#   show port info 0
#   set fwd txonly
#   start
#   show port stats all
#   quit

# Build the custom application
cd /root/dpdk_workspace/Task_3_EAL_Port_Init/
chmod +x scripts/*.sh
./scripts/build.sh

# Run the custom application
export DPRC=dprc.2
./src/kyocera_pmd_init -l 0-3 -n 1 --file-prefix=kpmd

Document Approval

| Role | Name | Date |
| --- | --- | --- |
| Created By | Nandini & Mary | 13-04-2026 |
| Reviewed By | Aswanth | 14-04-2026 |
| Approved By | Deepak | 15-04-2026 |

Tessolve Semiconductors | Kyocera SLD Laser | Confidential